|  | ## CT-2000PRO/PRO-i/PLUS |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Special Model for Textile/Spinning Industry. |  |  |  |  |  |  |  |  |
|  | Characteristic. Output frequency 0.5~240.0Hz. External analog setting 0~10V and 4~20mA. Automatic torque compensation and slip compensation. PWN frequency 1.5~10KHz. 10 pattern and any V/F pattern. Built-in PI control. Built-in standard RS422/485 communication port, MODBUS RTU protocol. 8-step speed setting. Frequency increase/decrease（UP / DOWN）function and 16-step function control setting. Restart / Free run start. Operation interface (LCD & LED dual display for Pro, LED display for Plus). 2-step acceleration/deceleration. Built-in specialized functions for textile industry. |  |  |  |  |  |  |
|  | Standard Specification A. CT2000Plus 200V Series Motor rating (KW); 2.2; 3.7; 5.5; 7.5; 11; 15; 22; 30; 37; 45; 55; 75; 93. Model CT2000Plus-; 2A2; 3A7; 5A5; 7A5; 011; 015; 022; 030; 037; 045; 055; 075; 093. Rated current (A); 11.1; 18; 23; 33; 48; 61; 86; 125; 150; 170; 210; 278; 330. Rated capacity (KVA); 4.4; 7.1; 9.2; 13.1; 19.1; 24.3; 34.3; 49; 60; 68; 84; 111; 131. Power supply; 3∮ 200~230V ±10% 50HZ/60Hz ±5%. Output voltage; 3∮ 200V、220V、230V. Cooling system; Forced air-cooling. Weight (Kg); 8; 8; 9; 14; 14; 22; 23; 46; 49; 50; 75; 83; 89. B. CT2000Plus 400V Series Motor rating (KW); 2.2; 3.7; 5.5; 7.5; 11; 15; 22; 25; 30; 37; 45; 55; 75; 93; 112. Model CT2000Plus-; 2A2; 3A7; 5A5; 7A5; 011; 015; 022; 025; 030; 037; 045; 055; 075; 093; 112. Rated current (A); 6.2; 9; 13; 17.3; 24; 31; 52; 61; 65; 71; 93; 110; 156; 180; 225. Rated capacity (KVA); 4.9; 7.1; 10.4; 13.8; 19.1; 24.7; 41.4; 47.1; 51.8; 56.6; 74; 87.6; 124; 143; 180. Power supply; 3∮ 380~460V ±10% 50HZ/60Hz ±5%. Output voltage; 3∮ 380V、400V、415V、440V、460V. Cooling system; Forced air-cooling. Weight (Kg); 8; 8; 9; 15; 15; 17; 25; 25; 26; 49; 51; 53; 79; 82; 91.. |  |